Unit 2

Introduction to Computer Networks

[5 Hrs]

2.1 Definitions, Uses, Benefits

2.2 Network Topologies and its types (Bus, Star, Ring, Mesh, Hybrid)

2.3 Types of Computer Networks (PAN, LAN, MAN, WAN, CAN)

2.4 Networking Types (Client/Server, P2P)

2.5 Overview of Protocols and Standards

2.6 OSI Reference Model

2.7 TCP/IP Models and its Comparison with OSI

2.8 Networking Hardware: NIC, Hub, Repeater, Switch, Bridge, Router

2.9 Basic Networking Commands

2.1 Definitions, Uses, Benefits

| Definition of Computer Network A computer network is a collection of two or more computers or devices connected together to share data, resources, and services. These devices may be connected using: - cables - fiber optics - Wi-Fi - radio waves When computers communicate and share resources with each other, it is called a computer network. Examples of a Computer Network - computers connected in a school lab - office computers sharing one printer - mobile phone using Wi-Fi internet - ATM network of banks - hospital computers sharing patient records Benefits of Computer Networks 1. Cost Saving Instead of buying separate printers, storage, and internet for each computer, devices can share them. 2. Easy Communication Users can exchange information quickly. 3. Better Data Management Data can be stored centrally and managed efficiently. 4. Increased Reliability Backup systems and multiple connections can improve reliability. 5. Scalability New devices can be added easily. 6. Collaboration Many users can work together on project | Uses of Computer Networks Computer networks are used for many purposes: 1. Resource Sharing Devices can share: - printer - scanner - storage - internet connection Example: In a college office, many computers may use one printer through a network. 2. Data Sharing Users can send and receive: - files - documents - photos - videos Example: A teacher sends notes to students through the college network. 3. Communication Networks help people communicate through: - email - chat - video call - voice call Example: A team meeting can be done using Zoom or Google Meet over a network. 4. Centralized Management Data and software can be controlled from one central server. Example: In a bank, all branch computers may connect to a central database server. 5. Remote Access Users can access information from distant places. Example: A person in Butwal can access a server in Kathmandu through the internet. |
| --- | --- |

2.2 Network Topologies

| Network topology means the physical or logical arrangement of computers and network devices in a network. In simple words, it shows how devices are connected. 1. Bus Topology In bus topology, all devices are connected to a single main cable called the backbone cable. Structure: All computers share one communication line. Example: A small old office network where all computers connect to one long cable. Advantages: - simple to install - low cost - less cable required Disadvantages: - if main cable fails, whole network fails - difficult to troubleshoot - performance decreases with more devices 2. Star Topology In star topology, all devices are connected to a central device such as a hub or switch. Structure: Each computer has a separate cable to the central device. Example: Most modern school and office LANs use star topology. Advantages: - easy to install and manage - failure of one cable affects only one device - easy to expand Disadvantages: - if central device fails, entire network fails - needs more cable | 3. Ring Topology In ring topology, each device is connected to two neighboring devices, forming a circular path. Structure: Data travels around the ring. Example: Older token ring networks. Advantages: - orderly data transmission - less collision Disadvantages: - failure in one node/cable may affect whole network - difficult to reconfigure 4. Mesh Topology In mesh topology, every device is connected to every other device. Structure: Multiple paths exist between devices. Example: Critical military or research communication systems. Advantages: - very reliable - high fault tolerance - secure communication Disadvantages: - very expensive - large amount of cabling - difficult installation 5. Hybrid Topology A hybrid topology is a combination of two or more topologies. Example: A college network may use star topology inside each department and combine departments in another form. Advantages: - flexible - scalable - can combine strengths of different topologies Disadvantages: - complex design - higher cost |
| --- | --- |

2.3 Types of Computer Networks

| Computer networks can be classified according to their size and coverage area. 1. PAN – Personal Area Network A PAN is a small network around one person. Range: Very short distance, usually a few meters. Examples: - Bluetooth connection between phone and earbuds - phone connected to laptop hotspot Use: Personal devices communication. 2. LAN – Local Area Network A LAN connects computers within a small area such as: - room - home - office - school lab Example: A computer lab in a college. Features: - high speed - private ownership - low cost | 3. MAN – Metropolitan Area Network A MAN covers a city or town. Example: A network connecting different offices of a company across Butwal city. Features: - larger than LAN - smaller than WAN 4. WAN – Wide Area Network A WAN covers a large geographical area such as: - country - continent - world Example: The internet is the best example of WAN. Features: - connects distant networks - may use leased lines, fiber, satellite 5. CAN – Campus Area Network A CAN connects multiple LANs within a campus or institution. Example: A university network connecting administration block, library, labs, and hostels. Features: - larger than LAN - smaller than MAN common in colleges and hospitals |
| --- | --- |

2.4 Networking Types (Client/Server, P2P)

| 1. Client/Server Network In a client/server network, one or more computers act as servers, and others act as clients. Server A server provides: - files - database - printer service - web service Client A client requests services from the server. Example: In a bank, employees’ computers act as clients and connect to a central database server. Advantages: - centralized control - better security - easy backup - efficient management Disadvantages: - expensive - server failure affects many users - requires administration | 2. Peer-to-Peer (P2P) Network In a peer-to-peer network, all computers are equal. Each computer can act as both client and server. Example: Two computers sharing files directly in a small office. Advantages: - simple - low cost - no dedicated server needed Disadvantages: - less security - difficult management - not suitable for large networks |
| --- | --- |

2.5 Overview of Protocols and Standards

| Protocol A protocol is a set of rules used for communication between devices. Protocols define: - how data is formatted - how it is sent - how it is received - how errors are handled Protocols are rules for communication between network devices. Standards ensure compatibility among devices and technologies. Both are necessary for successful computer networking. Examples: - HTTP - HTTPS - FTP - TCP - IP - SMTP Standard A standard is an agreed rule or guideline followed by manufacturers and organizations to ensure compatibility. Example: Wi-Fi standards allow devices from different companies to connect. | Why Protocols and Standards are Important - ensure communication between different devices - make networking reliable - support interoperability - reduce confusion - improve compatibility Examples of Common Protocols HTTP Used for web browsing. HTTPS Secure version of HTTP. FTP Used for file transfer. SMTP Used for sending email. TCP Provides reliable data delivery. IP Handles addressing and routing. |
| --- | --- |

2.6 OSI Reference Model

| The OSI Model stands for Open Systems Interconnection Model. It is a conceptual framework that explains how data moves from one device to another through a network. It has 7 layers. The OSI reference model has seven layers: Physical, Data Link, Network, Transport, Session, Presentation, and Application. Each layer has a specific function and helps in structured network communication. 1. Physical Layer Responsible for transmitting raw bits through the medium. Functions: - cables - connectors - signal transmission Example: Ethernet cable, hub 2. Data Link Layer Responsible for node-to-node delivery and error detection. Functions: - framing - MAC addressing - error detection Example: Switch, NIC 3. Network Layer Responsible for logical addressing and routing. Functions: - IP addressing - routing Example: Router | 4. Transport Layer Responsible for end-to-end delivery. Functions: - segmentation - flow control - error control Example: TCP, UDP 5. Session Layer Responsible for establishing, managing, and terminating sessions. Example: Login session between systems. 6. Presentation Layer Responsible for data translation, encryption, and compression. Example: Data format conversion, SSL 7. Application Layer Closest to the user; provides network services to applications. Examples: HTTP, FTP, SMTP, DNS |
| --- | --- |

2.7 TCP/IP Models and its Comparison with OSI

| Introduction The TCP/IP model is a networking model used in real computer networks, especially on the Internet. It explains how data moves from one device to another. TCP/IP stands for: - TCP = Transmission Control Protocol - IP = Internet Protocol The TCP/IP model is simpler than the OSI model and has 4 layers. Layers of TCP/IP Model 1. Application Layer This layer provides services directly to user applications. It combines the functions of: - Application layer - Presentation layer - Session layer of the OSI model. Examples of protocols: - HTTP - HTTPS - FTP - SMTP - DNS Functions: - file transfer - email - web browsing - remote login | 2. Transport Layer This layer is responsible for end-to-end communication between source and destination. Examples of protocols: - TCP - UDP Functions: - segmentation - flow control - error control - reliable delivery 3. Internet Layer This layer is responsible for logical addressing and routing. Example of protocol: - IP Functions: - IP addressing - packet routing - path selection 4. Network Access Layer This is the lowest layer of TCP/IP model. It handles actual transmission of data through the physical network. It combines the functions of: - Data Link layer - Physical layer of the OSI model. Functions: - framing - MAC addressing - transmission of bits - error detection at local link Comparison Between TCP/IP and OSI OSI Model; TCP/IP Model. 7 layers; 4 layers. Theoretical model; Practical model. Developed by ISO; Developed by DARPA. Session and Presentation are separate; Session and Presentation are included in Application layer. Physical and Data Link are separate; Combined as Network Access layer. Used for understanding network concepts; Used in real Internet communication. |
| --- | --- |

2.8 Networking Hardware:

Networking hardware are the physical devices used to build and operate a computer network. These devices help computers communicate, send data, receive data, and connect different parts of a network.

The main networking hardware are:

- NIC

- Hub

- Repeater

- Switch

- Bridge

- Router

1. NIC (Network Interface Card)

A NIC is a hardware component that allows a computer or device to connect to a network.

It may be:

- wired NIC

- wireless NIC

Functions:

- connects computer to network

- provides physical address called MAC address

- sends and receives data

Example:

- Ethernet card in desktop computer

- Wi-Fi adapter in laptop

Simple example:

If a computer wants to connect to the internet through LAN cable, it needs a NIC.

2. Hub

A hub is a simple networking device that connects multiple computers in a network.

When a hub receives data from one device, it sends that data to all connected devices.

Functions:

- connects multiple devices

- broadcasts data to all ports

Advantages:

- simple

- low cost

Disadvantages:

- sends data to all devices

- causes more traffic

- less secure

- less efficient

Example:

Old small office networks used hubs to connect computers.

3. Repeater

A repeater is a device that regenerates or strengthens weak signals.

When data travels a long distance, the signal becomes weak. A repeater refreshes the signal and sends it again.

Functions:

- receives weak signal

- regenerates signal

- extends transmission distance

Example:

If a LAN cable is very long, a repeater can be used to extend the network.

4. Switch

A switch is a networking device that connects multiple computers in a LAN and sends data only to the correct destination device.

Unlike a hub, a switch does not broadcast to all devices unnecessarily.

Functions:

- connects multiple devices

- forwards data to specific device

- uses MAC address table

Advantages:

- faster than hub

- less traffic

- more secure

- better performance

Example:

Modern computer labs and offices use switches.

Simple example:

If computer A sends data to computer B, the switch sends it only to B, not to all computers.

5. Bridge

A bridge is a device that connects two LAN segments and filters network traffic.

It works at the data link layer and uses MAC addresses.

Functions:

- connects two parts of a network

- reduces unnecessary traffic

- filters data

Example:

If one office network is divided into two parts, a bridge can connect them.

Advantage:

- reduces collision

- improves performance

6. Router

A router is a networking device that connects different networks and routes data packets from one network to another.

It works using IP addresses.

Functions:

- connects different networks

- chooses best path for data

- routes packets

- connects LAN to internet

Example:

Home Wi-Fi router connects your home network to the internet.

Simple example:

If your laptop accesses a website, the router forwards your request from local network to the internet.

2.9 Basic Networking Commands

Basic networking commands are used to:

WIN + R , CMD

- check network connection

- test communication between devices

- view IP address

- troubleshoot internet problems

- check routes, DNS, and ports

These commands are very useful for students, network administrators, and computer users.

A. Basic Networking Commands in Windows

1. ipconfig

This command shows the basic IP configuration of the computer.

Use:

- view IP address

- view subnet mask

- view default gateway

Command:

ipconfig

Example output:

- IPv4 Address: 192.168.1.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

Purpose:

Used to know the current network settings of the computer.

2. ipconfig /all

This shows detailed network information.

Command:

ipconfig /all

Shows:

- IP address

- MAC address

- DNS server

- DHCP enabled/disabled

- lease information

Purpose:

Used when more detailed network configuration is needed.

3. ping

This command checks whether another device or website is reachable.

Command:

ping google.com

or

ping 192.168.1.1

Purpose:

- test connectivity

- check whether network is working

- measure response time

Example:

If reply comes, the destination is reachable.

4. tracert

This command shows the path taken by packets from source to destination.

Command:

tracert google.com

Purpose:

- find route path

- locate delay or failure point in network

5. nslookup

This command finds the IP address of a domain name.

Command:

nslookup google.com

Purpose:

- test DNS working

- convert domain name to IP address

6. arp -a

This command shows the ARP table.

Command:

arp -a

Purpose:

- see IP address and MAC address mapping

- troubleshoot LAN communication

7. netstat

This command shows active network connections and listening ports.

Command:

netstat

Common form:

netstat -an

Purpose:

- see open ports

- see active connections

- troubleshoot services and connections

8. hostname

This command shows the computer name.

Command:

hostname

Purpose:

Used to know the host/computer name in the network.

9. getmac

This command shows the MAC address of the system.

Command:

getmac

Purpose:

Used to identify physical network address.

10. pathping

This command combines features of ping and tracert.

Command:

pathping google.com

Purpose:

- check packet loss

- analyze route performance

B. Basic Networking Commands in Linux

..........................................killerkoda.com

1. ifconfig

This command shows or configures network interface information.
In some modern Linux systems, ifconfig may not be installed by default.

Command:

ifconfig

Purpose:

- see IP address

- check network interface details

2. ip a

This is the modern Linux command for viewing IP address information.

Command:

ip a

or

ip addr

Purpose:

- show IP address

- show interface status

- display network interface details

3. ping

Same as Windows, used to test connectivity.

Command:

ping google.com

Note:

In Linux, ping runs continuously until stopped with Ctrl + C.

Purpose:

- test reachability

- test network response

4. traceroute

This command shows the route packets take to reach destination.

Command:

traceroute google.com

Purpose:

- view path

- detect routing problem

5. nslookup

Used to find DNS information.

Command:

nslookup google.com

Purpose:

- test DNS resolution

- find IP from domain name

6. dig

This is another DNS lookup command in Linux.

Command:

dig google.com

Purpose:

- get detailed DNS information

- query DNS records

7. arp

Shows ARP table.

Command:

arp -a

Purpose:

- display IP to MAC mapping

8. netstat

Shows connections, routes, interface statistics.

Command:

netstat -an

Purpose:

- display active connections

- show listening ports

9. ss

This is a modern replacement for netstat in Linux.

Command:

ss -tuln

Purpose:

- show open ports

- show socket information

10. hostname

Shows the system hostname.

Command:

hostname

Purpose:

Used to identify system name.

11. route

Displays routing table.

Command:

route -n

Purpose:

- view routing path

- troubleshoot routing issue

12. ip route

Modern command to show routing table.

Command:

ip route

Purpose:

- display route entries

- check gateway and route path