Unit 7

Advanced Microprocessors

(9 Hrs.)

8086: logical block diagram and segments,

80286: Architecture, Registers, (Real/Protected mode), Privilege levels, descriptor cache, Memory access in GDT and    LDT, multitasking, addressing modes, flag register

80386: Architecture, Register organization, Memory access in protected mode, Paging

Intel 8086

🔷 Introduction

- The Intel 8086 is a 16-bit microprocessor developed by Intel in 1978.

- It is the successor to the 8085 microprocessor and was a major step forward in microprocessor architecture.

- It introduced pipelining, segmented memory, and 16-bit data and address buses, enabling more powerful and efficient computing.

- It became the foundation for the x86 architecture, which is still widely used in modern computing systems.

🔷 Back with of Intel 8085 and 8086

| Feature | 8085 | 8086 |
| --- | --- | --- |
| Data Bus | 8-bit | 16-bit |
| Address Bus | 16-bit (64 KB) | 20-bit (1 MB) |
| ALU Size | 8-bit | 16-bit |
| Pipelining | No | Yes |
| Registers | Fewer | More (and wider) |
| Operating Modes | Single mode | Min/Max mode |
| Memory Segmentation | No | Yes |
| Instruction Set | Basic | Advanced |

Features of 8086

- 16-bit data bus & 20-bit address bus

- Pipelined architecture (fetch & execute simultaneously)

- 14 registers (general-purpose, segment, pointer, index, flag)

- Operating frequency: 5 MHz (8086), 8 MHz (8086-2), 10 MHz (8086-1)

- Multiplexed address/data bus (AD0–AD15)

- Supports hardware interrupts (NMI & INTR)

- Two modes of operation:

  - Minimum mode (single processor)

  - Maximum mode (multiprocessor)

Why Pipeline?

In computing, pipelining improves the efficiency of instruction execution by allowing multiple instructions to overlap in execution—like an assembly line in a factory.

Basic Concept:

When an application runs:

APP → List of Programs → Instructions → Process → Thread

Each instruction (I) typically goes through four stages:

- F: Opcode Fetch

- D: Decode

- E: Execute

- S: Store

Let’s consider 4 instructions: I1, I2, I3, I4

🔹 For Single Processor (No Pipelining):

Each instruction is executed one after the other:

Time to execute = I1 + I2 + I3 + I4

= (F1 + D1 + E1 + S1) + (F2 + D2 + E2 + S2) +

(F3 + D3 + E3 + S3) + (F4 + D4 + E4 + S4)

= 4 ns + 4 ns + 4 ns + 4 ns

= **16 ns**

🔹 For Parallel Processor (With Pipelining):

Instructions are overlapped using pipeline stages:

| Time to execute = I1 + I2 + I3 + I4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | F1 | D1 | E1 | S1 |  |  |  |
| P2 |  | F2 | D2 | E2 | S2 |  |  |
| P3 |  |  | F3 | D3 | E3 | S3 |  |
| P4 |  |  |  | F4 | D4 | E4 | S4 |

| Time to execute = I1 + I2 + I3 + I4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | F1 | D1 | E1 | S1 |  |  |  |
| P2 |  | F2 | D2 | E2 | S2 |  |  |
| P3 |  |  | F3 | D3 | E3 | S3 |  |
| P4 |  |  |  | F4 | D4 | E4 | S4 |
|  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Total Time = 7 cycles × 1 ns (per stage) = **7 ns**

Cycle 1: F1

Cycle 2: D1   F2

Cycle 3: E1   D2   F3

Cycle 4: S1   E2   D3   F4

Cycle 5:      S2   E3   D4

Cycle 6:           S3   E4

Cycle 7:                S4

Total Time = 7 cycles × 1 ns (per stage) = **7 ns**

✅ Benefit of Pipelining:

- Non-pipelined execution: 4 instructions → 16 ns

- Pipelined execution: 4 instructions → 7 ns

- Time saved: 9 ns

- Efficiency increased by overlapping instruction stages.

Pin Diagram of the Intel 8086 Microprocessor

Intel 8086 Microprocessor, which consists of 40 pins. Each pin serves a specific function depending on whether the processor is operating in minimum mode (single processor system) or maximum mode (multiprocessor system).

🔹 Power & Clock Pins

- Vcc (Pin 40): +5V power supply.

- GND (Pins 1, 20): Ground connection.

- CLK (Pin 19): Clock input. Provides timing for internal operations.

- RESET (Pin 21): Resets the processor and sets program counter to zero.

🔹 Address/Data Bus (Multiplexed)

- AD0 – AD15 (Pins 2–16, 39):
These are multiplexed lines used to carry address (A0–A15) during the first part of the bus cycle and data (D0–D15) during the second part.

🔹 Address Bus (High)

- A16/S3 – A19/S6 (Pins 35–38):
These are multiplexed with status signals (S3–S6) and used for addressing (A16–A19) in memory access.

🔹 Control & Status Pins

- ALE (Pin 25): Address Latch Enable. Indicates when AD0–AD15 contains an address.

- DEN (Pin 26): Data Enable. Enables the external data bus transceivers.

- DT/R (Pin 27): Data Transmit/Receive. Controls direction of data flow.

- M/IO (Pin 28): Distinguishes memory or I/O operation.

- RD (Pin 32): Read control signal.

- WR (Pin 29): Write control signal.

- READY (Pin 22): Indicates if peripheral is ready for data transfer.

- TEST (Pin 23): Used for synchronization with external devices.

🔹 Interrupt Pins

- INTR (Pin 18): Interrupt request (maskable).

- NMI (Pin 17): Non-maskable interrupt. Has higher priority than INTR.

🔹 Minimum/Maximum Mode Selection

- MN/MX̅ (Pin 33):

  - Logic 1 → Minimum mode

  - Logic 0 → Maximum mode

🔹 Minimum Mode Specific Pins

(used only when MN/MX̅ = 1)

- INTA (Pin 24): Interrupt acknowledge.

- HOLD (Pin 31): DMA controller requests control of buses.

- HLDA (Pin 30): DMA controller granted access to buses.

- WR, RD, ALE, DEN, DT/R, M/IO → Active in this mode.

🔹 Maximum Mode Specific Pins

(used only when MN/MX̅ = 0)

- S2̅, S1̅, S0̅ (Pins 26–28): Status lines for controlling bus cycles.

- RQ/GT1̅, RQ/GT0̅ (Pins 34, 35): Request/Grant for bus control in multiprocessor mode.

- LOCK (Pin 29): Ensures exclusive access to shared resources.

- QS1, QS0 (Pins 24, 25): Queue status signals.

Intel 8086 architecture

The block diagram of the Intel 8086 architecture, showing the internal structure of the microprocessor. The 8086 is divided into two main units:

- Bus Interface Unit (BIU)

- Execution Unit (EU)

🧠 1. Bus Interface Unit (BIU)

Key Responsibilities:

- Handles address generation, instruction fetching, and communication with memory and I/O.

- Feeds the Execution Unit (EU) with a stream of instructions.

Components:

- Segment Registers (ES, CS, SS, DS)
These hold segment addresses for Code (CS), Data (DS), Stack (SS), and Extra segment (ES).

- Instruction Pointer (IP)
Points to the next instruction in the Code Segment (CS).

- Adder (Σ)
Combines segment base and offset to form the physical address.

- Instruction Queue
Pre-fetches up to 6 bytes of instructions (using pipelining). This allows the Execution Unit to execute instructions faster.

- Memory Interface
Controls the communication between memory and processor using the address and data buses.

⚙️ 2. Execution Unit (EU)

Key Responsibilities:

- Decodes and executes instructions

- Performs arithmetic and logic operations

- Controls flags and operands

Components:

- General Purpose Registers
Used for data storage and manipulation:

  - AH/AL, BH/BL, CH/CL, DH/DL → Can be used as 8-bit or combined into AX, BX, CX, DX (16-bit)

  - SP (Stack Pointer), BP (Base Pointer), SI (Source Index), DI (Destination Index)

- Arithmetic and Logic Unit (ALU)
Performs all arithmetic (add, subtract, etc.) and logic (AND, OR, NOT, etc.) operations.

- Operands and Flags

  - Operands: Data inputs to the ALU.

  - Flags: Special-purpose bits reflecting the outcome of ALU operations (e.g., Zero, Carry, Overflow).

🔄 Data Buses:

- A-Bus:
Transfers data between registers and ALU in the Execution Unit.

- C-Bus:
Transfers addresses and instructions from the BIU to memory and instruction queue.

⛓️ Summary:

| Component | Purpose |
| --- | --- |
| BIU | Handles memory interaction, instruction fetching |
| Segment Registers | Holds memory segment addresses |
| Instruction Pointer | Points to next instruction |
| Instruction Queue | Stores pre-fetched instructions (6 bytes max) |
| EU | Executes instructions using ALU |
| General Registers | Temporary storage and data manipulation |
| ALU | Arithmetic and logic processing |
| Flags | Reflect results of operations |
| A-Bus, C-Bus | Internal data and control signal pathways |

Registers in 8086

General Purpose Registers (8)

Pointer and Index Registers (4)

Segment Registers (4)

Instruction Pointer (IP)

Flag Register (1 total)

A. General Purpose Registers (8)

Used for arithmetic, logic, data transfer operations.

| Register | 16-bit | 8-bit Parts | Usage |
| --- | --- | --- | --- |
| AX | AH + AL | AH, AL | Accumulator |
| BX | BH + BL | BH, BL | Base register |
| CX | CH + CL | CH, CL | Counter (loops, shifts) |
| DX | DH + DL | DH, DL | Data register (I/O, MUL/DIV) |

B. Pointer and Index Registers (4)

| Register | Function |
| --- | --- |
| SP | Stack Pointer |
| BP | Base Pointer |
| SI | Source Index (used in string operations) |
| DI | Destination Index |

C. Segment Registers (4)

Used to access different memory segments (20-bit address space)

| Register | Segment |
| --- | --- |
| CS | Code Segment (for IP) |
| DS | Data Segment |
| SS | Stack Segment |
| ES | Extra Segment |

🔸 These segment registers hold the base addresses used to compute the 20-bit physical address.

D. Instruction Pointer (IP)

- Holds offset of the next instruction in the code segment (CS).

E. Flag Register (1 total)

- 16-bit register with 9 active flags (explained below)

Physical Address Calculation in 8086

8086 uses Segment: Offset addressing, combining a 16-bit segment register and a 16-bit offset to form a 20-bit physical address.

Physical Address = (Segment × 10h) + Offset

Example: Let’s say:

Segment = 1234h			Offset  = 0010h

Then,

Physical Address = (1234h × 10h) + 0010h

= 12340h + 0010h

= **12350h**

Flags Register in 8086 (16-bit)

A. Status Flags

Indicate results of operations:

- CF – Carry Flag

- PF – Parity Flag

- AF – Auxiliary Carry Flag

- ZF – Zero Flag

- SF – Sign Flag

- OF – Overflow Flag

B. Control Flags

Control processor operations:

- TF – Trap Flag (for debugging)

- IF – Interrupt Enable Flag

- DF – Direction Flag (string operations)

Addressing Modes in 8086

An addressing mode specifies how an operand (data) for an instruction is accessed (i.e., whether it is in a register, memory, or part of the instruction itself).

8086 supports several addressing modes to allow flexible memory and data access.

1. Immediate Addressing Mode

- The operand is directly specified in the instruction.

- Data is given as a constant value.

MOV AX, 1234h     ; AX ← 1234h

Use: Simple data loading.

2. Register Addressing Mode

- The operand is located in a register.

- Both source and destination are registers.

MOV AX, BX        ; AX ← BX

Use: Fast data movement within CPU.

3. Direct Addressing Mode

- The memory address of the operand is given explicitly in the instruction.

- Offset is fixed; segment is usually DS by default.

MOV AX, [1234h]   ; AX ← contents at memory address DS:1234h

Use: Access fixed memory location.

4. Register Indirect Addressing Mode

- The effective address of the operand is stored in a register.

- Registers used: BX, BP, SI, or DI.

- Segment: DS for BX, SI, DI; SS for BP.

MOV AX, [BX]      ; AX ← contents at DS:BX

Use: Access variable memory locations dynamically.

5. Based Addressing Mode

- A base register holds the offset (BX or BP).

- Optional displacement can be added.

- Segment: DS (for BX), SS (for BP)

MOV AX, [BP]      ; AX ← contents at SS:BP

Use: Access stack or data memory.

6. Indexed Addressing Mode

- Uses index registers: SI or DI

- Effective address = contents of index register

- Segment default: DS

MOV AX, [SI]      ; AX ← contents at DS:SI

Use: Access array elements.

7. Based-Indexed Addressing Mode

- Combines base register (BX or BP) and index register (SI or DI).

- Effective address = base + index

- Segment default: DS or SS depending on base

MOV AX, [BX + SI] ; AX ← contents at DS:(BX+SI)

Use: Multidimensional arrays or struct access.

8. Based-Indexed with Displacement

- Most flexible addressing mode.

- Effective address = base + index + displacement

- Allows full offset control.

MOV AX, [BX + SI + 10] ; AX ← contents at DS:(BX + SI + 10)

Use: Array element + offset access.

9. Relative Addressing Mode

Used only with jump (JMP) and branch instructions.

- The target address is relative to the current instruction pointer (IP).

JMP SHORT LABEL    ; IP ← IP + displacement to LABEL

Use: Efficient code branching.

| Mode | Syntax Example | Effective Address Formula |
| --- | --- | --- |
| Immediate | MOV AX, 1234h | Operand is in instruction |
| Register | MOV AX, BX | Operand in register |
| Direct | MOV AX, [1234h] | EA = 1234h |
| Register Indirect | MOV AX, [BX] | EA = contents of BX |
| Based | MOV AX, [BP] | EA = BP |
| Indexed | MOV AX, [SI] | EA = SI |
| Based + Indexed | MOV AX, [BX + SI] | EA = BX + SI |
| Based + Indexed + Disp | MOV AX, [BX + SI + 5] | EA = BX + SI + 5 |
| Relative (Jumps only) | JMP SHORT LABEL | New IP = Old IP + displacement |

Programming with 8085

Software Simulator: https://github.com/sanjeevlcc/notes_2081/blob/main/Microprocessor_and_CA/TU%20BSC-CSIT%20Microprocessor/emu8086v408.zip

After Running program

Sample 8086 Addition

Addition (8-bit)

MOV AL, 25H    ; Load 25H into AL

MOV BL, 15H    ; Load 15H into BL

ADD AL, BL     ; AL = AL + BL → AL = 25H + 15H = 3AH

Subtraction (8-bit)

MOV AL, 25H    ; Load 25H into AL

MOV BL, 15H    ; Load 15H into BL

SUB AL, BL     ; AL = AL - BL → AL = 25H - 15H = 10H

Addition (16-bit)

MOV AX, 1234H  ; Load 1234H into AX

MOV BX, 1111H  ; Load 1111H into BX

ADD AX, BX     ; AX = AX + BX → AX = 1234H + 1111H = 2345H

Subtraction (16-bit)

MOV AX, 1234H  ; Load 1234H into AX

MOV BX, 1111H  ; Load 1111H into BX

SUB AX, BX     ; AX = AX - BX → AX = 1234H - 1111H = 0123H

Multplication (8-bit)

MOV AL, 05H     ; AL = 5

MOV BL, 04H     ; BL = 4

MUL BL          ; Unsigned multiplication: AL × BL → result in AX

; Result: AL = 14H (20 decimal), AH = 00H (no overflow)

Division (8-bit)

MOV AL, 14H     ; AL = 20

MOV BL, 04H     ; BL = 4

DIV BL          ; Unsigned division: AX / BL → quotient in AL, remainder in AH

; Quotient: AL = 05H, Remainder: AH = 00H

Multplication (16-bit)

MOV AX, 0012H   ; AX = 0012H (18 decimal)

MOV BX, 0003H   ; BX = 0003H (3 decimal)

MUL BX          ; Unsigned multiplication: AX × BX → result in DX:AX

; Result: AX = 0036H (54), DX = 0000H (no overflow)

Division (16-bit)

MOV AX, 0037H   ; Dividend low word

MOV DX, 0000H   ; Dividend high word (clear DX)

MOV BX, 0003H   ; Divisor = 3

DIV BX          ; Unsigned division: DX:AX / BX

; Quotient: AX = 0012H (0B), Remainder: DX = 0001H

Simple “Hello World” Program

.MODEL SMALL

.STACK 100H

.DATA

MSG DB 'Hello, World!$'  ; Message ends with '$' for DOS string termination

.CODE

MAIN:

MOV AX, @DATA

MOV DS, AX              ; Initialize data segment

LEA SI, MSG             ; Load offset of MSG into SI

PRINT_LOOP:

MOV AL, [SI]            ; Load character from MSG into AL

CMP AL, '$'             ; Check for end of string

JE EXIT                 ; If '$', exit loop

MOV DL, AL              ; Move character to DL

MOV AH, 02H             ; DOS function: print character

INT 21H                 ; Call DOS interrupt

INC SI                  ; Move to next character

JMP PRINT_LOOP          ; Repeat

EXIT:

MOV AH, 4CH             ; DOS terminate program

INT 21H

END MAIN

OR

; HELLO.ASM - Hello World program for 8086

; Uses DOS INT 21h function 09h to print a string

; The string must be terminated with a '$' character

.MODEL SMALL

.STACK 100H

.DATA

MESSAGE DB 'Hello, World!','$'  ; Define message with $ terminator

.CODE

MAIN PROC

; Initialize data segment

MOV AX, @DATA

MOV DS, AX

; Display message using DOS function 09h

MOV AH, 09h          ; DOS function for string output, keyboard

;;LEA DX, MESSAGE    ; Load effective address of message

INT 21h              ; Call DOS interrupt

; Return to DOS

MOV AH, 4Ch          ; DOS function for program termination

INT 21h              ; Call DOS interrupt

MAIN ENDP

END MAIN

Palindrome

.MODEL SMALL

.STACK 100H

.DATA

; Define the test string (you can change this)

STRING      DB 'MADAM', '$'      ; String to check (must be uppercase)

STR_LEN     EQU 5                ; Length of the string (excluding $)

; Output messages

MSG_YES     DB 'Palindrome$', 0

MSG_NO      DB 'Not Palindrome$', 0

.CODE

MAIN PROC

; Set up segments

MOV AX, @DATA

MOV DS, AX

; Setup SI and DI to compare characters

LEA SI, STRING            ; SI -> start of the string

LEA DI, STRING

ADD DI, STR_LEN - 1       ; DI -> end of the string (before $)

MOV CX, STR_LEN / 2       ; Number of comparisons (half length)

MOV BX, 0                 ; BX = mismatch flag

CHECK_LOOP:

MOV AL, [SI]

MOV DL, [DI]

CMP AL, DL

JNE NOT_PALINDROME

INC SI

DEC DI

LOOP CHECK_LOOP

; If no mismatches found

LEA DX, MSG_YES

JMP PRINT_RESULT

NOT_PALINDROME:

LEA DX, MSG_NO

PRINT_RESULT:

MOV AH, 09H               ; DOS function to print string

INT 21H

; Exit program

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

Subtraction (16-bit)

80286: Architecture, Registers, (Real/Protected mode), Privilege levels, descriptor cache, Memory access in GDT and    LDT, multitasking, addressing modes, flag register

✅ Summary Table

| Bus Type | Width | Direction | Purpose |
| --- | --- | --- | --- |
| Address | 24-bit | Unidirectional | Specifies memory/I/O addresses |
| Data | 16-bit | Bidirectional | Transfers data and instructions |
| Control | Varies | Mixed | Controls operations like read/write |

| Feature | Description |
| --- | --- |
| Address Bus | 24-bit → 16MB memory |
| Real Mode | 8086-compatible, 1MB memory, no protection |
| Protected Mode | Enables multitasking, memory protection |
| GDT/LDT | Define memory segments for system and tasks |
| Multitasking | Through Task State Segment and task descriptors |
| Descriptor Cache | Internal registers holding descriptor info |
| Privilege Levels | 0 (kernel) to 3 (user) security levels |
| Flags Register | Status and control flags for arithmetic, execution |

✅ 1. Address Bus 🔹 Definition: - The Address Bus is used by the processor to specify memory addresses or I/O ports to read from or write to. 🔹 Key Features: - Width: 24-bit address bus - Addressable Memory: 2²⁴ = 16 MB of physical memory - Unidirectional: Data flows only from CPU to memory/I/O 🔹 Use: - Sends memory addresses to RAM, ROM, or I/O devices.

✅ 2. Data Bus 🔹 Definition: - The Data Bus carries actual data values between the CPU, memory, and I/O devices. 🔹 Key Features: - Width: 16-bit data bus - Bidirectional: Data flows both ways (to and from the CPU) 🔹 Use: - Transfers instructions, operand values, and I/O data. - 16 bits at a time → suitable for word operations.

✅ 3. Control Bus 🔹 Definition: - The Control Bus carries control signals that manage and coordinate all CPU activities. Signal; Description. RD̅; Read – Memory or I/O read operation. WR̅; Write – Memory or I/O write operation. ALE; Address Latch Enable – Separates address/data. DT/R̅; Data Transmit/Receive – Bus direction. DEN̅; Data Enable – Enables external data bus. READY; Wait state insertion from slow devices. INTA̅; Interrupt Acknowledge. NMI; Non-maskable interrupt. LOCK; Locks system bus for atomic operations. HOLD/HLDA; Bus control for DMA.

✅ 1. Architecture Overview

- Introduced by Intel in 1982, 80286 (or iAPX 286) is a 16-bit microprocessor.

- Clock speed: 6 MHz – 25 MHz.

- 24-bit address bus → Can address up to 16 MB of physical memory.

- Supports Real Mode (8086-compatible) and Protected Mode (advanced features).

✅ 2. Registers of 80286

🔹 General-Purpose Registers (16-bit):

- AX (Accumulator)

- BX (Base)

- CX (Count)

- DX (Data)

🔹 Segment Registers:

- CS (Code Segment)

- DS (Data Segment)

- SS (Stack Segment)

- ES (Extra Segment)

🔹 Pointer and Index Registers:

- SP (Stack Pointer), BP (Base Pointer)

- SI (Source Index), DI (Destination Index)

🔹 Instruction Pointer and Flags:

- IP (Instruction Pointer)

- FLAGS (Status and control bits)

✅ 3. Real Mode vs Protected Mode

| Feature | Real Mode | Protected Mode |
| --- | --- | --- |
| Addressable Memory | 1 MB (20-bit addressing) | 16 MB physical, 1 GB virtual memory |
| Mode Type | Compatible with 8086 | Advanced features enabled |
| Multitasking | Not supported | Supported |
| Protection | None | Memory protection, privilege levels |
| Switching | Default on reset | Entered by setting PE (CR0 bit 0) |

✅ 4. Privilege Levels

- 4 Privilege Levels (Ring 0 to Ring 3):

  - Ring 0: Most privileged (Kernel/OS)

  - Ring 3: Least privileged (User apps)

- Used for memory and I/O protection.

- CPL (Current Privilege Level) vs DPL (Descriptor Privilege Level).

✅ 5. Descriptor Cache (Shadow Registers)

- Internal registers that store segment descriptor info (base, limit, access rights) from memory.

- When a segment is loaded (e.g., MOV DS, AX), the CPU fetches the descriptor from GDT/LDT and stores in descriptor cache.

- Fast access to segment properties without repeatedly accessing memory.

✅ 6. Memory Access with GDT and LDT

🔹 Global Descriptor Table (GDT)

- System-wide table with segment descriptors.

- Each entry defines: Base address, Limit, Access rights.

- Used for code, data, stack segments, etc.

🔹 Local Descriptor Table (LDT)

- Specific to each task/process.

- Defines memory accessible to that task.

- Enables task isolation and memory protection.

✅ 7. Multitasking

- 80286 supports hardware-level multitasking using the Task State Segment (TSS).

- Each task has:

  - Own register set, LDT, stack, flags.

  - Described using a Task Descriptor in GDT.

- Task Switching involves:

  - Saving current task state to its TSS.

  - Loading new task state from next TSS.

  - Updating Task Register and control flags.

✅ 8. Addressing Modes in 80286

Supports all 8086 addressing modes:

🔹 Register and Immediate Modes:

- MOV AX, BX

- ADD AX, 1234H

🔹 Memory Addressing Modes:

- Direct: [1234H]

- Register Indirect: [BX], [SI], [DI]

- Based: [BX+SI], [BX+DI]

- Based Indexed + Displacement: [BX+SI+10H]

→ Uses Effective Address (EA) computation:
EA = Base + Index + Displacement

✅ 9. Flag Register (FLAGS)

| Bit | Flag Name | Description |
| --- | --- | --- |
| 0 | CF (Carry) | Carry out from MSB |
| 2 | PF (Parity) | Set if even number of 1’s in LSB |
| 4 | AF (Auxiliary) | Carry from bit 3 to bit 4 |
| 6 | ZF (Zero) | Set if result is zero |
| 7 | SF (Sign) | Set if result is negative |
| 8 | TF (Trap) | Enables single-step mode |
| 9 | IF (Interrupt) | Enables/disables hardware interrupts |
| 10 | DF (Direction) | Controls string operation direction |
| 11 | OF (Overflow) | Set if signed overflow occurs |

Intel 80386 Microprocessor

Architecture

Register organization

Memory access in protected mode

Paging

| Feature / Processor | 8086 | 80186 | 80286 | 80386 |
| --- | --- | --- | --- | --- |
| Year Introduced | 1978 | 1982 | 1982 | 1985 |
| Architecture | 16-bit | 16-bit | 16-bit | 32-bit |
| Data Bus Width | 16-bit | 16-bit | 16-bit | 32-bit |
| Address Bus Width | 20-bit (1 MB memory) | 20-bit (1 MB memory) | 24-bit (16 MB memory) | 32-bit (4 GB memory) |
| Clock Speed (typical) | 5 – 10 MHz | 6 – 10 MHz | 6 – 25 MHz | 12 – 40 MHz |
| Instruction Set | Basic x86 | 8086 + extended instructions | 8086 + Protected Mode | Full x86 + paging and 32-bit ops |
| Registers | 16-bit general-purpose | Same as 8086 | Same as 8086 | 32-bit registers (EAX, EBX, etc.) |
| Operating Modes | Real Mode only | Real Mode only | Real Mode, Protected Mode | Real, Protected, and Virtual 8086 Mode |
| Segmentation Support | Yes | Yes | Yes | Yes + Descriptor Tables |
| Paging Support | No | No | No | Yes (2-level) |
| Virtual Memory | No | No | Limited (segment-based) | Yes (paging-based) |
| Multitasking Support | No | No | Partial (via TSS) | Yes (hardware support) |
| Integrated Peripherals | No | Yes (timer, DMA, interrupt) | No | No |
| Memory Protection | No | No | Yes (via segment descriptors) | Yes (segment + page protection) |
| Privilege Levels | None | None | 4 levels (0–3) | 4 levels (0–3) |
| Real Mode Compatibility | Native | Native | Compatible | Compatible |
| Applications | Early PCs | Embedded systems | Early multitasking OSs | Advanced OSs, multitasking systems |

1. Architecture of 80386

- The Intel 80386 is a 32-bit microprocessor introduced in 1985.

- It supports multitasking, memory protection, and virtual memory.

- Operates in three modes:

  - Real Mode (8086 compatible)

  - Protected Mode (advanced mode with full features)

  - Virtual 8086 Mode (for multitasking legacy software)

🔷 Architectural Highlights:

- 32-bit data and address bus.

- Can address 4 GB of physical memory (2³²).

- Supports segmentation and paging.

- Internal pipelining for faster execution.

- Integrated Memory Management Unit (MMU).

- Clock speed: 12–40 MHz.

2. Register Organization in 80386

Intel 80386 is a 32-bit microprocessor with an enhanced register set over 8086/80286, supporting both 16-bit and 32-bit operations. The registers are categorized into:

🔹 1. General Purpose Registers (GPRs)

These are used for arithmetic, logical operations, data movement, and addressing. Each 32-bit register is backward compatible with its 16-bit and 8-bit counterparts.

| 32-bit |  | 16-bit | 8-bit High | 8-bit Low |
| --- | --- | --- | --- | --- |
| EAX | EAX: Accumulator (used in arithmetic) | AX | AH | AL |
| EBX | EBX: Base register (often used in memory addressing) | BX | BH | BL |
| ECX | ECX: Counter (used in loops, shift/rotate) | CX | CH | CL |
| EDX | EDX: Data register (used with EAX in multiplication/division) | DX | DH | DL |
| ESI | ESI/EDI: Source/Destination Index (used in string operations) | SI | – | – |
| EDI |  | DI | – | – |
| EBP | EBP: Base Pointer (for stack frame access) | BP | – | – |
| ESP | ESP: Stack Pointer (points to top of the stack) | SP | – | – |

🔸 b. Segment Registers (16-bit):

Used to select segments (Code, Data, Stack, Extra) via descriptors.

- CS: Code Segment

- DS: Data Segment

- SS: Stack Segment

- ES: Extra Segment

- FS, GS: Additional segments (used for OS/multitasking)

These are used with segment selectors to access memory via the descriptor tables in Protected Mode.

🔸 c. Instruction Pointer:

EIP (Extended Instruction Pointer):
32-bit register that holds the offset address of the next instruction to be executed.

🔸 d. Flags Register:

Stores status flags, control flags, and system flags.

Common Flags:

- CF – Carry Flag

- PF – Parity Flag

- AF – Auxiliary Carry

- ZF – Zero Flag

- SF – Sign Flag

- TF – Trap Flag

- IF – Interrupt Enable Flag

- DF – Direction Flag

- OF – Overflow Flag

Additional system flags support virtual memory and protection.

🔸 e. Control Registers (CR0–CR3):

Used to control the processor mode, enable/disable paging, and manage memory protection.

| Register | Purpose |
| --- | --- |
| CR0 | Enables Protected Mode (bit 0 = PE), Paging (bit 31 = PG) |
| CR2 | Stores the linear address that caused a page fault |
| CR3 | Holds the base address of the Page Directory (used in paging) |

🔸 f. System Descriptor Registers:

Used for memory segmentation and interrupt handling in Protected Mode.

| Register | Purpose |
| --- | --- |
| GDTR | Global Descriptor Table Register – base and limit of GDT |
| LDTR | Local Descriptor Table Register – for tasks |
| IDTR | Interrupt Descriptor Table Register – base/limit of IDT |
| TR | Task Register – points to Task State Segment (TSS) |

🔹 7. Debug and Test Registers (Advanced Use)

- DR0–DR7: Debug Registers – for hardware breakpoints.

- TR6–TR7: Test Registers – used internally for cache testing.

✅ 3. Memory Access in Protected Mode (Intel 80386)

🔹 What is Protected Mode?

Protected Mode is an advanced operating mode of the Intel 80386 (and 80286+) that supports:

- Full 32-bit memory addressing

- Multitasking

- Memory protection

- Virtual memory via segmentation and paging

It allows access to the entire 4 GB address space using segmentation and optional paging.

🔹 Memory Access Mechanism in Protected Mode

Memory access in protected mode is done in three steps:

🔸 Step 1: Logical Address = Segment Selector + Offset

- The logical address provided by the program consists of:

  - A segment selector (16-bit)

  - A logical offset (32-bit)

- The segment selector identifies an entry in either:

  - Global Descriptor Table (GDT)

  - Local Descriptor Table (LDT)

🔸 Step 2: Selector → Descriptor → Linear Address

- The segment selector is used to index into GDT/LDT to get the segment descriptor.

- The segment descriptor contains:

  - Base Address (32-bit)

  - Limit (size of the segment)

  - Access Rights (read, write, execute, privilege level)

- The Base Address from the descriptor is added to the offset to form the Linear Address.

Linear Address = Base (from descriptor) + Offset

🔸 Step 3 (Optional): Linear Address → Physical Address (via Paging)

- If paging is enabled, the linear address is translated to a physical address using:

  - Page Directory (pointed to by CR3)

  - Page Tables

Otherwise, the linear address is used directly as the physical address.

🔹 Example Summary Flow

Logical Address = Selector + Offset

↓

Use Selector → GDT/LDT → Descriptor

↓

Get Base Address → Add Offset → Linear Address

↓

(Optional) Linear Address → Paging → Physical Address

4. Paging in 80386

🔹 What is Paging?

Paging is a memory management scheme that allows the operating system to:

- Use virtual memory

- Divide memory into fixed-size pages

- Isolate processes

- Protect memory from illegal access

In 80386, paging can be used with or without segmentation, and provides a way to map linear addresses to physical addresses.

🔹 Key Features of Paging in 80386

- Page size: 4 KB (4096 bytes)

- Addressable memory: 4 GB

- Two-level paging structure

- Enables virtual memory and process isolation

- Controlled via Control Registers (CR0, CR3)

🔹 Address Translation in Paging

80386 uses a 2-level page table to translate a 32-bit linear address into a physical address.

🧠 Breakdown of 32-bit Linear Address:

| Bits | Used For |
| --- | --- |
| 31–22 | Page Directory Index (10 bits) |
| 21–12 | Page Table Index (10 bits) |
| 11–0 | Offset within page (12 bits) |

🔸 Paging Data Structures

a. Page Directory (1024 entries)

- Each entry points to a Page Table

- Size: 4 bytes per entry × 1024 = 4 KB

b. Page Tables (1024 entries each)

- Each entry points to a 4 KB Page Frame in physical memory

- Size: 4 bytes per entry × 1024 = 4 KB

c. Page Frame

- The actual physical memory block (4 KB) that stores data

🔸 Address Translation Process

- CR3 holds the base address of Page Directory

- Extract bits from the linear address:

  - Bits 31–22: Page Directory Index → Get Page Table Address

  - Bits 21–12: Page Table Index → Get Page Frame Address

  - Bits 11–0: Offset → Final Byte within Page Frame

- Combine Page Frame Base + Offset → Physical Address

📌 Summary:

Linear Address (32-bit)

↓

[Directory Index] → Page Directory → Page Table

↓

[Table Index] → Page Table Entry → Page Frame

↓

[Offset] → Final Byte in Physical Memory

🔸 Enabling Paging in 80386

Paging is enabled by setting:

- Bit 31 (PG) of CR0 to 1 (Paging Enable)

- CR3 must point to the Page Directory Base

Example (in conceptual pseudo-code)

MOV CR3, PageDirectoryBase

SET CR0.PG = 1     ; Enable Paging

🔹 Advantages of Paging

| Benefit | Description |
| --- | --- |
| Virtual Memory | Allows programs to use more memory than RAM |
| Protection | Isolates processes; prevents memory leaks |
| Efficiency | Allocates only used memory in 4 KB chunks |
| Swapping | Enables demand paging & page swapping |

🔹 Comparison with Segmentation

| Feature | Segmentation | Paging |
| --- | --- | --- |
| Division Type | Logical (varied-size segments) | Physical (fixed-size pages) |
| Addressing | Selector + Offset | Linear → Physical (via tables) |
| Flexibility | Better for logical structures | Better for virtual memory |

✅ Summary Table:

| Feature | Description |
| --- | --- |
| Address Bus | 32-bit (4 GB addressable) |
| Modes | Real, Protected, Virtual 8086 |
| Key Registers | EAX–EDI, EIP, EFLAGS, Segment Registers |
| Control Registers | CR0–CR3 |
| Memory Mgmt | Segmentation + Paging |
| Paging Unit | Two-level (Page Directory + Page Table) |
| Page Size | 4 KB |